/*

---------------------------------------------------------------------------------

Código de Calibração de uma unidade de sensor

---------------------------------------------------------------------------------

Este programa implementa a calibração do parâmetro de classificação de uma cor para um sensor.

O procedimento de calibração consiste em sensoriar uma face com cor conhecida e executando este código

obter HUE_Médio para determinada cor que servirá de parametro de classificação nos sensoriamentos futuros.

Cada um dos 12 sensores deve ter 6 valores de HUE_Médio, um para cada cor.

Logo o programa de calibração completa (a ser implementado) deve obter uma matriz 12x6 com 96 valores.

*/

#include <Wire.h>

#include "TCS34725.h"

#include <string.h>

TCS34725 tcs;

#define LED_0 2

#define LED_1 3

#define LED_2 4

#define LED_3 5

#define LED_4 6

#define LED_5 7

#define LED_6 8

#define LED_7 9

#define LED_8 10

#define LED_9 11

#define LED_10 12

#define LED_11 13

#define EN_MUX 23

#define MUX_0 22

#define MUX_1 18

#define MUX_2 17

#define MUX_3 15

#define AMOSTRAS 20

struct RGBsum {

float red;

float green;

float blue;

float sum;

};

struct HSV {

float hue;

float sat;

float val;

};

void setup() {

Serial.begin(115200);

Wire.begin();

tcs.integrationTime(33);

tcs.gain(TCS34725::Gain::X01);

pinMode(LED_0, OUTPUT);

pinMode(LED_1, OUTPUT);

pinMode(LED_2, OUTPUT);

pinMode(LED_3, OUTPUT);

pinMode(LED_4, OUTPUT);

pinMode(LED_5, OUTPUT);

pinMode(LED_6, OUTPUT);

pinMode(LED_7, OUTPUT);

pinMode(LED_8, OUTPUT);

pinMode(LED_9, OUTPUT);

pinMode(LED_10, OUTPUT);

pinMode(LED_11, OUTPUT);

pinMode(EN_MUX, OUTPUT);

pinMode(MUX_0, OUTPUT);

pinMode(MUX_1, OUTPUT);

pinMode(MUX_2, OUTPUT);

pinMode(MUX_3, OUTPUT);

}

void loop() {

Serial.println("Calibrar qual sensor? (digite número)");

while (Serial.available() <= 0) {

// Aguarda confirmação de movimento pela serial

}

while (Serial.available() > 0){ // Enviado qualquer confirmação pela serial continua

int NS = Serial.read(); // limpa serial

}

// Leitura LED desligado

MUX(NS,0);

auto ref = sensor_rgb();

// Leitura LED ligado

MUX(NS,1);

auto abs = sensor_rgb();

// Retira fator luz externa

float r_NS = abs.red - ref.red;

float g_NS = abs.green - ref.green;

float b_NS = abs.blue - ref.blue;

float sum_NS = abs.sum - ref.sum;

r_NS /= sum_NS;

g_NS /= sum_NS;

b_NS /= sum_NS;

// Converte RGB para HSV

auto v_color = rgbToHsv(r_NS, g_NS, b_NS);

Serial.print("Hue medido é ");

Serial.println(v_color.hue);

}

// Funções Sensor

RGBsum sensor_rgb() {

if (!tcs.attach(Wire)) {

Serial.print("Sensor nao encontrado!");

// Aguarda Sensor conectar

while (1);

}

delay(4000); // alterar delay?

RGBsum cor;

cor.red = 0;

cor.green = 0;

cor.blue = 0;

cor.sum = 0;

int i = 0;

while (i < AMOSTRAS) {

while (!tcs.available());

auto c = tcs.color();

float r = c.r, g = c.g, b = c.b;

float soma = r + g + b;

if (soma < 5) {continue;}

i++;

cor.red += r;

cor.green += g;

cor.blue += b;

cor.sum += soma;

}

return cor;

}

HSV rgbToHsv(float r, float g, float b) {

float maxc = max(r, max(g, b));

float minc = min(r, min(g, b));

float delta = maxc - minc;

HSV u;

u.val = maxc;

if ((maxc == 0)||(delta == 0)) { u.sat = 0; u.hue = 0; return u; }

else if (maxc == r) u.hue = 60.0 * fmod(((g - b) / delta), 6);

else if (maxc == g) u.hue = 60.0 * (((b - r) / delta) + 2);

else u.hue = 60.0 * (((r - g) / delta) + 4);

if (u.hue < 0) u.hue += 360.0;

u.sat = delta / maxc;

return u;

}

void MUX(int n,bool flag_LED){

digitalWrite(EN_MUX, LOW);

digitalWrite(LED_0, LOW);

digitalWrite(LED_1, LOW);

digitalWrite(LED_2, LOW);

digitalWrite(LED_3, LOW);

digitalWrite(LED_4, LOW);

digitalWrite(LED_5, LOW);

digitalWrite(LED_6, LOW);

digitalWrite(LED_7, LOW);

digitalWrite(LED_8, LOW);

digitalWrite(LED_9, LOW);

digitalWrite(LED_10, LOW);

digitalWrite(LED_11, LOW);

if (flag_LED) { //Liga o LED só se flag_LED==true

switch(n) {

case 1:

digitalWrite(LED_1, HIGH);

break;

case 2:

digitalWrite(LED_2, HIGH);

break;

case 3:

digitalWrite(LED_3, HIGH);

break;

case 4:

digitalWrite(LED_4, HIGH);

break;

case 5:

digitalWrite(LED_5, HIGH);

break;

case 6:

digitalWrite(LED_6, HIGH);

break;

case 7:

digitalWrite(LED_7, HIGH);

break;

case 8:

digitalWrite(LED_8, HIGH);

break;

case 9:

digitalWrite(LED_9, HIGH);

break;

case 10:

digitalWrite(LED_10, HIGH);

break;

case 11:

digitalWrite(LED_11, HIGH);

break;

default:

digitalWrite(LED_0, HIGH);

} // Liga o LED do sensor NS

}

// Seleciona o Sensor

if ((n & (1<<0)) == (1<<0)) {digitalWrite(MUX_0, HIGH);}

else{digitalWrite(MUX_0, LOW);}

if ((n & (1<<1)) == (1<<1)) {digitalWrite(MUX_1, HIGH);}

else{digitalWrite(MUX_1, LOW);}

if ((n & (1<<2)) == (1<<2)) {digitalWrite(MUX_2, HIGH);}

else{digitalWrite(MUX_2, LOW);}

if ((n & (1<<3)) == (1<<3)) {digitalWrite(MUX_3, HIGH);}

else{digitalWrite(MUX_3, LOW);}

//Seleciona Sensor NS pelo MUX

digitalWrite(EN_MUX, HIGH);  // Habilita MUX

}